This assignment must be hand-written. Show ALL steps in ALL questions

[UNGRADED tasks are highly recommended for practice, but you do not need to submit them]

- Design a circuit diagram for the following system that takes two 4-bit binary numbers [A and B] as inputs and outputs in the following fashion:

- If one input is odd and the other input is even, the output should be A+B

- If both inputs are odd or both inputs are even, the output should be A-B

- [Ungraded] Using a 4 bit parallel adder, perform A-B if B is exactly 8 or 14. Otherwise perform A+B. Assume that both A and B are 4 bit numbers.

- [Ungraded] Design a 13 people attendance system using necessary parallel and half adder(s). Your circuit diagram should be efficient, i.e. use the least amount of components.

- Design a 16 people voter system using necessary parallel and full adder(s). Your circuit diagram should be efficient, i.e. use the least amount of components.

- Design a BCD to excess 5 system using necessary parallel adder(s).

- Construct the function F(a,b,c,d) = ∑(0, 1, 2, 5, 7, 9, 10, 15):

- Using only 3x8 decoders

- Using 3x8 decoder(s) and 2x4 decoder(s) (you need to use both in the same circuit)

- Using only 2x4 decoders

You may use basic gates in every case. Your circuit must be efficient, which means you need to use the least amount of components.

- Construct the function F(a,b,c,d,e) = ∑(0,1, 5, 8, 9, 13, 16, 19, 24, 25, 31):

- Using 16x1 and 2x1 mux(s) (you need to use both in the same circuit)

- Using only 8x1 muxes. You cannot use any basic gates.

- Using 8x1 and 4x1 mux(s) (you need to use both in the same circuit)

Your circuit must be efficient, which means you need to use the least amount of components.

- Construct the following functions using a single 8x1 mux and necessary basic gates:

- F(a,b,c) = ∑(1, 3, 4, 5, 7)

- F(a,b,c,d) = ∑(0, 2, 3,  7, 8,  9, 10, 11, 15)

- Construct the following circuits using only 4x1 mux(s):

- ab’ + c

- (x’+y)(y+z’) [Ungraded]

- [Ungraded] Design a 3-bit parallel adder where using appropriate encoders and decoders to build the internal circuit.

- [Ungraded] You have to construct the truth table and internal circuit diagram for a  4x2 priority encoder. The priority is given in the following manner:

- If the number of 1 in a binary input is even, then priority will be given to the MSB line.

- If the number of 1 in a binary input is odd, then priority will be given to the LSB line.

- [Ungraded] Design a 16x1 mux using only a single 4x1 mux. You may use other basic gates as required.